RS Slot Gasket Kit, HypaBond

You Will Need:

1 inch Chisel

400 Grade Sand Paper

Cloth or Rag

Gloves

Kit Includes:

RS Gasket

Tradesolve4

HypaBond

Tradecure 100

- Remove old slot gasket (and the two front screws of the bailer moulding if it is an early RS200/400).

- Remove old Sikaflex (or old adhesive) using the chisel (be careful not to damage the hull)!

- Once all the Sikaflex has been removed, lightly roughen the recessed surface with sand paper.

- Wipe and clean surface with the Tradesolve 4 but do not use all of it.

- Put the gloves on and mix the HypaBond and Tradecure 100 together. You should use all of both pots, as this will give you the correct mix ratio. If you are feeling more confident the mix ratio is 3%.

- Apply the mixed glue to both the hull and the bumpy side of the slot gasket.

- Allow to dry for 10 minutes before putting the gasket onto the hull.

- Starting at the bow end place the new slot gasket into position, sliding the gasket under the bailer moulding (RS200 and 400 only).

- When you are happy that the gasket is in the correct position, apply pressure until the gasket is fair with the hull.

- Wipe away all excess glue using the Tradesolve 4, trying not to get it on the gasket itself and allow to dry. For best results allow 12 hours for the adhesive to go off.

- Then remove tape and go sailing!