RS500 Righting Line

# Kit Contains:	Part Numbers:

# 2 x P Clips 							A4035

# 2x P Clips (Drilled to 6mm for Shroud U Bolt)		A4035

2 x 6mm Dome Nuts					S1

2 x M5 Nyloc Nuts						F141

2 x M5 x 30mm Csk Screws				F006

2 x 1.75m 8-Plait Pre-Stretched Rope (4mm)		JB006

2 x 0.75m Shockcord (5mm)				WHSAC415

INSTRUCTIONS:

Place a P clip over the aft part of the shroud u-bolt (found under the gunwale) and fix in place using a dome nut.

Drill through the gunwale 750mm aft of the U-bolt and countersink the hole.

Using the F006 screws and the F141 nyloc nuts, fix a P clip under the gunwale.

Following the gunwale around the transom lip, drill a hole 30mm down from the deck and drill another hole a further 20mm (50mm total) down the transom lip.

Attach the one of the pieces of rope to the P clip at the shroud U-bolt and thread the other end through the other P clip.

Looping the shockcord through the 2 holes at the transom, attach it to the free end of the rope.

Repeat for the other side.