- Cut kicker control line to 8.10m and remove any twists.

- Locate the small deck eye with ring equal distance between the back of the thwart and mainsheet base (Pict. 1).

- Using a 2.5mm drill bit mark and drill the fixing holes. Countersink and inject with Sikaflex.

- Attach the deck eye and ring with 6 x ½ pozi pan self tappers using a PZ2 screw driver. Wipe away any excess Sikaflex.

- Using a 3.5mm drill bit drill a hole in each grab rail in line with the back of the thwart (Pict. 2).

- Tie a ring to each grab rail using the 2mm rope provided. With an overhand knot above and below (Pict. 3).

- Rope out as normal. Once through the cleats thread the line down through the grab rail ring, under the helm toe strap and finally through the centre ring on the C/B case (Pict. 4 & 5).

- Splice ends together as per the instruction sheet.

Continuous Kicker Fit out.

1. 2.

3. 4.

5.

Tools required

Drill

2.5 & 3.5mm drill bits

Countersink

PZ2 bit/screw driver

Sealant gun

Fixings used

| Fixing | Size | Qty |
| --- | --- | --- |
| Pozi pan self tapper | 6 x 1/2 | 2 |

PDI No:

Skill Level

QC Notes

- Cut control line to correct length.

- Ensure all twists are removed before splicing.