RS Q’BA CREW TOE STRAP INSTRUCTIONS

CONTENTS OF PACK

1 X SET OF INSTRUCTIONS

2 X CREW TOE STRAPS

1 X 30CM LENGTH OF 4MM ROPE

- USING A 4MM ALLEN KEY, REMOVE ALL 4 BOLTS & 2 BLACK PLATES WHICH ATTACH ONE OF THE SIDE TOE STRAPS TO THE SIDE OF THE COCKPIT.

- REPLACE THE OLD TOE STRAP WITH ONE OF THE NEW CREW TOE STRAPS.

- LINE THE 4 FIXING HOLES UP WITH THE INSERTS ON THE COCKPIT SIDE (INSURE THE RS LOGO IS FACING INTO THE BOAT AND TOWARDS THE TRANSOM).

- USING THE 4MM ALLEN KEY AGAIN, SCREW IN ALL 4 BOLTS (WITH BLACK PLATES). DO NOT OVER TIGHTEN; HAND TIGHT IS MORE THAN ENOUGH.

- REPEAT THE PROCESS ON THE OPPOSITE SIDE.

- THREAD THE 30CM LENGTH OF ROPE THROUGH BOTH LOOPS ON THE END OF THE TOE STRAPS AND SECURE TO THE DECK FIXING AT THE FRONT OF THE COCKPIT.